What is a computer?

A computer is an electronic machine that accepts input, processes it according to instructions, stores it, and produces output. This is called the IPO cycle (Input, Process, Output), with Storage as the fourth step.

Example: you type a letter on the keyboard (input), the CPU handles it (process), the file is saved to the SSD (storage), and the letter appears on the monitor (output).

A computer is fast, accurate, and can store huge amounts of data. It has no intelligence of its own and only follows the instructions it is given.

What is hardware?

Hardware is every physical part of a computer that you can see and touch. It falls into four groups:

- Input devices: keyboard, mouse, scanner, microphone, webcam.

- Processing devices: CPU (the brain), motherboard (connects everything), GPU (graphics), RAM (temporary working memory).

- Storage devices: HDD, SSD, USB drive, optical disc.

- Output devices: monitor, printer, speakers, projector.

Other hardware supports these parts, such as the power supply (PSU), cooling fans, cables and the case. Hardware can also be internal (inside the case, such as the motherboard and RAM) or external (peripherals, such as the keyboard and printer).

What is software?

Software is the set of instructions and programs that tell the hardware what to do. You cannot touch it. It is stored on the drive and runs in memory. There are three main types:

- System software: runs and manages the computer itself.

  - Operating systems: Windows, macOS, Linux, Android, iOS.

  - Firmware: BIOS/UEFI on the motherboard.

  - Drivers: let the OS talk to devices such as printers and graphics cards.

  - Utilities: antivirus, disk cleanup, backup tools.

- Application software: helps users do tasks, such as MS Word, Excel, Chrome, VLC, Photoshop and games.

- Programming software: used to create other software, such as compilers, code editors and IDEs (VS Code, Python, Visual Studio).

Hardware vs software

|  | Hardware | Software |
| --- | --- | --- |
| Nature | Physical, touchable | Logical, not touchable |
| Made of | Electronic and mechanical parts | Code and instructions |
| Can be damaged by | Dust, heat, water, shocks | Viruses, bugs, corruption |
| Fixed by | Repair or replacement | Update, reinstall, patch |
| Examples | CPU, RAM, monitor | Windows, Chrome, BIOS |

Day 1: Motherboard Components

Motherboard kya hota hai?

Motherboard computer ka main circuit board hai. CPU, RAM, hard disk/SSD, graphics card, fan, sab kuch ya to isi par lagta hai ya isse connect hota hai. Isi ke zariye saare parts ek dusre se baat karte hain.

Example: CPU dimaag hai, to motherboard nervous system aur skeleton dono hai. Uske bina saare parts alag-alag pade rahenge aur kaam nahi karenge.

Form factor (size aur shape)

Form factor ka matlab board ka size aur screw holes ki position. Case aur board ka form factor match hona chahiye.

- ATX: standard size, 7 slots tak. Gaming aur normal desktop mein use hota hai.

- Micro-ATX: chhota, 4 slots tak. Office aur budget PC mein use hota hai.

- Mini-ITX: sabse chhota, sirf 1 slot. Chhote PC mein use hota hai.

- E-ATX: sabse bada. Workstation aur server mein use hota hai.

Chhota board bade case mein fit ho jata hai, par bada board chhote case mein nahi.

Motherboard ke important parts

- CPU socket: yahan processor lagta hai. LGA mein pins board par hote hain, PGA mein pins CPU par hote hain.

- Chipset: CPU, RAM, storage aur USB ke beech traffic police ki tarah kaam karta hai.

- RAM slots (DIMM): yahan memory modules lagte hain. DDR4 aur DDR5 aapas mein fit nahi hote.

- PCIe slots: graphics card, sound card aur network card ke liye. Sizes x1, x4, x8, x16 hote hain.

- SATA ports: HDD aur SSD ko connect karne ke liye (6 Gbps tak).

- M.2 slot: chhoti, fast SSD (NVMe) ke liye. SATA se kaafi tez hoti hai.

- BIOS/UEFI chip: isme firmware hota hai jo computer ko start karta hai.

- CMOS battery (CR2032, 3V): date, time aur BIOS settings yaad rakhti hai. Ye kharab ho jaye to time baar-baar reset hota hai.

- Power connectors: 24-pin main power ke liye, 4/8-pin CPU power ke liye.

- VRM: PSU ke 12V ko CPU ke liye kam aur stable voltage mein badalta hai.

- Fan headers: 3-pin (DC control) aur 4-pin (PWM control, better speed control).

- Front panel header: case ke power button, reset button aur LEDs yahan lagte hain.

- USB aur audio headers: front panel ke USB ports aur headphone/mic ke liye.

- I/O panel: peeche ke ports jaise USB, HDMI, LAN aur audio.

- TPM header: encryption ke liye (Windows 11 mein zaroori).

- Jumpers: CLEAR_CMOS jumper BIOS settings ko reset karta hai.

Software wala hissa

BIOS vs UEFI:

- BIOS purana hai: text interface, MBR (2 TB tak ki disk).

- UEFI naya hai: graphical interface, GPT (bade disk), Secure Boot, fast boot.

Power button dabane par kya hota hai:

- PSU “Power Good” signal bhejta hai.

- CPU firmware (BIOS/UEFI) chalata hai.

- POST (Power-On Self-Test) CPU, RAM aur video check karta hai. Problem ho to beep sunai deti hai.

- Firmware drives aur devices ko detect karta hai.

- Boot order ke hisaab se boot device chuna jata hai.

- Bootloader OS ko load karta hai.

- OS drivers load karta hai aur aap desktop dekhte hain.

Motherboard ka model kaise dekhein (bina kholke):

- Windows: msinfo32 ya wmic baseboard get product,manufacturer

- Linux: sudo dmidecode -t baseboard

Common problems aur unke reasons

- Time reset ho raha hai: CMOS battery kharab.

- Fans chal rahe hain par display nahi: RAM ya CPU power ki problem.

- Lagatar beeps: memory detect nahi ho rahi.

- Load mein system band ho jata hai: overheating ya weak PSU.

Exam ke liye yaad rakhein

- LGA mein pins board par, PGA mein pins CPU par.

- Mini-ITX sabse chhota form factor hai.

- UEFI mein GPT, Secure Boot aur graphical interface hota hai.

- CMOS battery CR2032, 3V ki hoti hai.

- 24-pin main power aur 4/8-pin CPU power ke liye hota hai.

Day 2: Processors (simple explanation)

What is a CPU?

The CPU (Central Processing Unit) is the brain of the computer. It runs program instructions, does calculations, makes decisions and controls the flow of data between all other parts. It is a small silicon chip with billions of transistors.

Analogy: the computer is a kitchen. The CPU is the chef, RAM is the counter where ingredients are placed, and the program is the recipe.

What is inside a CPU?

- ALU (Arithmetic Logic Unit): does maths (add, subtract) and logic (compare, AND, OR).

- CU (Control Unit): directs everything by fetching and decoding instructions.

- Registers: tiny, extremely fast storage for the data being used right now.

- Cache: fast memory that keeps frequently used data close, so the CPU doesn’t wait for RAM.

The instruction cycle

Every instruction goes through four steps, billions of times per second:

- Fetch: get the instruction from memory.

- Decode: translate it into signals the CPU understands.

- Execute: carry out the operation.

- Store: save the result.

Key characteristics

- Clock speed (GHz): cycles per second. 4.0 GHz means 4 billion cycles per second. Compare it only within the same architecture.

- Cores: independent processing units on one chip. More cores mean better multitasking and heavy workloads.

- Threads: virtual cores. With Hyper-Threading (Intel) or SMT (AMD), one core handles two threads. This gives roughly 20-30% extra throughput, not double.

- Cache (L1, L2, L3): L1 is smallest and fastest (per core), L3 is largest and shared by all cores.

- Turbo/Boost: automatically raises speed on a few cores when more performance is needed.

- TDP (watts): the heat the cooler must remove, for example 65 W or 125 W.

- Process node (nm): manufacturing size. Smaller usually means cooler and more efficient.

- Socket: the physical connector, such as LGA1700 or AM5. It must match the motherboard.

- Integrated graphics: a GPU built into the CPU. It is fine for office work and video, but not for heavy gaming.

32-bit vs 64-bit

- 32-bit: can use only about 4 GB of RAM and runs only 32-bit software. It is obsolete.

- 64-bit: supports far more RAM and runs both 64-bit and 32-bit apps. It is the standard today.

On Windows, 64-bit systems have two folders: Program Files and Program Files (x86).

x86 vs ARM

- x86/x64 (CISC): used by Intel and AMD in desktops, laptops and servers. It is powerful but uses more power.

- ARM (RISC): used in phones, tablets and Apple M-series chips. It is very power efficient.

CISC means Complex Instruction Set Computing and RISC means Reduced Instruction Set Computing.

Intel and AMD families

|  | Intel | AMD |
| --- | --- | --- |
| Entry | Celeron, Pentium | Athlon |
| Mainstream | Core i3, i5, i7, i9 | Ryzen 3, 5, 7, 9 |
| Workstation | Core i9 | Threadripper |
| Server | Xeon | EPYC |

Reading a model name: in Core i7-14700K, i7 is the tier, 14 is the generation and K means unlocked for overclocking. F means no integrated graphics and U/P/H marks laptop chips. In Ryzen 7 7800X3D, X3D means extra stacked cache.

Intel 12th generation and later use P-cores (fast) and E-cores (efficient), and the OS assigns tasks to the right type.

Software side

- Virtualization: Intel VT-x and AMD-V let you run virtual machines. If a VM won’t start, enable this in BIOS/UEFI.

- NX/XD bit: blocks some malware by marking memory as non-executable.

- AES-NI: speeds up encryption such as BitLocker and VPNs.

- OS scheduler: decides which program runs on which core.

- Microcode updates: fix CPU bugs and security flaws through BIOS or Windows Update.

- Thermal throttling: if the CPU gets too hot, it slows itself down. This is a common cause of a lagging PC.

Check your CPU:

- Windows: Task Manager > Performance > CPU, or wmic cpu get name,numberofcores,numberoflogicalprocessors.

- Linux: lscpu or cat /proc/cpuinfo.

- Free tools: CPU-Z for specs, HWMonitor for temperatures.

Installing a CPU

- Lift the socket lever and match the triangle marker on the CPU with the socket.

- Place it gently without forcing it, then lock the lever.

- Apply a pea-sized amount of thermal paste and mount the cooler.

- Plug the fan into the CPU_FAN header and connect the 4/8-pin CPU power cable.

Common problems

- Shuts down under load: overheating, so reseat the cooler and clean dust.

- Very slow: thermal throttling or too many background programs.

- No display after a CPU upgrade: the BIOS may not support the new CPU, so update it.

- Virtual machine won’t start: virtualization is disabled in BIOS.

Exam points

- L1 is the fastest cache and L3 is the largest and shared.

- 32-bit supports about 4 GB of RAM.

- ARM is RISC and x86 is CISC.

- SMT is the AMD name for Hyper-Threading.

- TDP shows heat output, not exact power use

Day 3: Memory (simple explanation)

What is memory?

Memory is where the computer keeps the data and instructions the CPU needs right now or soon. It is organised in layers, from fastest and smallest to slowest and largest:

- Registers (inside the CPU)

- Cache (L1, L2, L3)

- RAM (main memory)

- SSD/HDD (permanent storage)

Analogy: the CPU is the chef, RAM is the kitchen counter (fast, limited space, cleared when you finish), and the SSD/HDD is the pantry (big, slower, permanent).

RAM vs ROM

|  | RAM | ROM |
| --- | --- | --- |
| Full form | Random Access Memory | Read-Only Memory |
| Volatile? | Yes, data is lost when power is off | No, data stays |
| Purpose | Holds running programs and open files | Holds startup instructions (firmware) |
| Example | DDR4 or DDR5 sticks | BIOS/UEFI chip |

ROM has variants: PROM (programmable once), EPROM (erasable with UV light), EEPROM (erasable electrically) and flash memory (what modern BIOS chips and SSDs use).

Types of RAM

- DRAM (Dynamic RAM): main system memory. It is cheap and dense but must be refreshed constantly.

- SRAM (Static RAM): very fast and expensive, and needs no refresh. It is used for CPU cache.

- SDRAM: DRAM synchronised with the system clock. All modern DDR memory is a type of SDRAM.

DDR generations

DDR means Double Data Rate: data moves on both the rising and falling edge of the clock.

| Type | Desktop DIMM pins | Voltage | Typical speed |
| --- | --- | --- | --- |
| DDR3 | 240 | 1.5 V | 1066-2133 MT/s |
| DDR4 | 288 | 1.2 V | 2133-3200+ MT/s |
| DDR5 | 288 | 1.1 V | 4800+ MT/s |

Each generation has its notch in a different position, so DDR4 will not fit a DDR5 slot. The CPU and motherboard decide which type you can use. DDR5 also has on-module power management and built-in on-die ECC.

Form factors

- DIMM: full-size stick for desktops and servers.

- SO-DIMM: small stick for laptops, mini PCs and some all-in-ones (260 pins for DDR4, 262 for DDR5).

Key characteristics

- Capacity: 8, 16, 32, 64 GB and more. A 32-bit OS can use only about 4 GB.

- Speed: measured in MT/s and often labelled DDR4-3200, or PC4-25600 (the number is bandwidth in MB/s).

- Latency (timings): values like CL16 or CL36 show how many clock cycles RAM waits before responding. Lower is better at the same speed.

- Channels: dual-channel uses two matched sticks in the correct slots to double the data path to the CPU. Quad-channel appears on workstations and servers.

- ECC (Error-Correcting Code): detects and fixes single-bit errors. It is used in servers and workstations and needs CPU and board support.

- Registered/buffered (RDIMM): adds a register chip for stability in large server memory configurations.

- XMP (Intel) / EXPO (AMD): profiles stored on the RAM. Enable them in BIOS/UEFI to reach the advertised speed, otherwise RAM runs at a lower default.

- Single-sided vs double-sided / ranks: chips on one or both sides of the stick. This affects capacity and compatibility.

Software side

- Virtual memory (page file/swap): when RAM is full, the OS uses part of the drive as extra memory. It is much slower, so heavy use makes the PC lag.

- File caching: the OS uses free RAM to keep frequently used files ready, so unused RAM is not wasted.

- Memory management: the OS gives each program its own memory space and stops programs from touching each other’s data.

- Memory leak: a buggy program keeps taking more RAM without releasing it, slowing the system until it is restarted.

Check your RAM:

- Windows: Task Manager > Performance > Memory (capacity, speed, slots used), or wmic memorychip get capacity,speed,manufacturer.

- Linux: free -h for usage, sudo dmidecode -t memory for stick details.

- Free tool: CPU-Z (Memory and SPD tabs).

Installing RAM

- Power off, unplug the PC and use an ESD strap.

- Open the slot clips and match the notch on the stick to the slot.

- Press down firmly on both ends until the clips click.

- For dual-channel with two sticks, use the colour-coded slots (usually A2 and B2). Check your manual.

- In BIOS/UEFI, confirm the total capacity and enable XMP/EXPO.

For laptops, SO-DIMMs go in at an angle and are pressed down until the side clips hold them.

Common problems

- No display or continuous beeps: RAM is badly seated or faulty. Reseat it and try one stick at a time.

- Random crashes or blue screens: faulty RAM or unstable XMP settings. Run Windows Memory Diagnostic or MemTest86.

- Less RAM shown than installed: a 32-bit OS, a stick not seated properly, or memory reserved for integrated graphics.

- System slow with high RAM use: too many programs or a memory leak. Check Task Manager and add more RAM if needed.

- PC will not boot after adding RAM: mismatched type, speed or voltage. Try each stick alone.

Exam points

- RAM is volatile; ROM is non-volatile.

- DRAM is main memory and SRAM is cache.

- DDR3 has 240 pins; DDR4 and DDR5 both have 288 pins on desktop DIMMs.

- SO-DIMM is for laptops.

- ECC corrects single-bit errors and is used in servers.

- Lower CL means lower latency.

- Virtual memory is slower than physical RAM.

Day 4: Cooling Systems (simple explanation)

Why do computers need cooling?

Every electronic part turns some of its power into heat. The CPU, GPU, VRM, RAM and drives all get hot when they work hard. Too much heat causes slowdowns, crashes, shortened component life and even permanent damage. A cooling system moves heat away from the parts and out of the case.

Analogy: think of running in a heavy jacket. You slow down and get tired. Cooling is taking the jacket off and having a breeze.

How heat moves

- Conduction: heat passes through direct contact, such as from the CPU into the heatsink.

- Convection: moving air or liquid carries heat away, as fans and coolant do.

- Radiation: heat spreads out as energy. It matters little in PCs.

Types of cooling

1. Air cooling (most common)

- Heatsink: a block of metal with fins, made of aluminium (cheap) or copper (better conductor). It absorbs heat from the chip and spreads it over a large surface.

- Fan: blows air through the fins so heat leaves quickly.

- Heat pipes: sealed copper tubes with a little liquid inside. The liquid evaporates at the hot end, travels to the cool end, condenses and returns, moving heat very efficiently.

- Types: the small stock cooler that comes with a CPU, and large tower coolers (single or dual tower) for better performance.

Pros: cheap, reliable, easy to install. Cons: big, and weaker on very hot CPUs.

2. Liquid cooling

- AIO (All-In-One): a sealed unit with a water block on the CPU, a pump, tubing and a radiator with fans. It needs no maintenance. Radiator sizes are 120, 240, 280 and 360 mm; bigger cools better.

- Custom loop: separate parts (pump, reservoir, tubing, blocks, radiator) assembled by the user. It is expensive, needs maintenance, and the best ones look and perform well.

Pros: better cooling for high-end CPUs, quieter at load, tidy look. Cons: costs more and a leak could damage the PC (rare in AIOs).

3. Passive cooling

No fan at all. A big heatsink relies on natural airflow. It is silent but only suitable for low-power chips, such as some mini PCs and fanless cases.

4. Extreme cooling (rare)

- Phase-change (like a refrigerator) and liquid nitrogen are used by overclockers for records, not for normal PCs.

Thermal paste and pads

Metal surfaces look flat but are microscopically rough, leaving tiny air gaps that block heat. Thermal paste fills those gaps.

- Use a pea-sized amount in the centre of the CPU. Too much or too little hurts cooling.

- Paste dries out over a few years and needs replacing.

- Thermal pads are soft squares used on VRMs, memory chips and M.2 drives.

- Some coolers come with paste pre-applied.

Case fans and airflow

- Common fan sizes: 80, 92, 120 and 140 mm. Larger fans move more air at lower, quieter speeds.

- Intake brings cool air in (front and bottom); exhaust pushes hot air out (rear and top), because hot air rises.

- Positive pressure: more intake than exhaust. It keeps dust out because air leaks out through gaps.

- Negative pressure: more exhaust than intake. It draws dust in through gaps.

- Keep cables tidy so they don’t block airflow.

- Leave space around desktops, and keep laptops on hard flat surfaces.

Key fan specifications

- RPM: rotations per minute. Higher is faster but louder.

- CFM: cubic feet per minute, the amount of air moved. Higher is better.

- Static pressure: how well a fan pushes air through resistance such as radiators and dust filters.

- Noise (dBA): lower is quieter.

- Connectors: a 3-pin fan has speed controlled by voltage (DC). A 4-pin fan has an extra wire for PWM, which gives finer, smoother control.

Other parts that need cooling

- GPU: has its own heatsink and fans, or a liquid unit.

- VRM and chipset: covered by small heatsinks.

- M.2 SSDs: can throttle when hot, so many boards include M.2 heatsinks.

- Power supply: has its own fan, which is usually temperature controlled.

- Laptops: use thin heat pipes and small blower fans, and they run hotter because space is tight.

Software side

- Temperature sensors: CPU and GPU have built-in sensors. Software reads them.

- Fan curves: you set fan speed against temperature in BIOS/UEFI or tools like the motherboard vendor utility, so fans stay quiet when idle and speed up under load.

- Thermal throttling: when the CPU or GPU reaches its limit (often around 95-100 C), it automatically lowers its speed to protect itself. If that is not enough, the PC shuts down.

- Monitoring tools: HWMonitor, HWiNFO, Core Temp, MSI Afterburner for GPU, and the temperature display in BIOS.

- Normal temperatures (rough guide): CPU idle 30-50 C; under load 60-85 C; sustained 90 C or more needs attention.

- Stress tests such as Cinebench or Prime95 help check if your cooling is enough, but run them with monitoring on.

Installation basics

- Power off, unplug the PC and use an ESD strap.

- Remove old paste with isopropyl alcohol and a lint-free cloth.

- Apply new paste (pea-sized) on the CPU.

- Mount the cooler evenly, tightening screws in a cross pattern.

- Plug the CPU fan into CPU_FAN (and an AIO pump into AIO_PUMP or CPU_FAN as the manual says).

- For an AIO, mount the radiator so the pump is not the highest point, to avoid air getting trapped.

- Check temperatures in BIOS on first boot.

Maintenance

- Clean dust every few months using compressed air. Hold fan blades still so they don’t over-spin.

- Check that all fans are spinning.

- Replace thermal paste every few years or when temperatures rise.

- Keep the PC in a well-ventilated place, away from walls and heaters.

Common problems

- Overheating or sudden shutdowns: dust, a failed fan, dried paste or poor airflow.

- Loud fans: dust, a failing bearing or an aggressive fan curve.

- High idle temperature: cooler badly seated, or too little or too much paste.

- CPU fan error at boot: the fan is not plugged into CPU_FAN, or it has failed.

- Gurgling or very hot AIO tubes: pump failure or trapped air. Stop using the PC and check it.

- Laptop gets very hot: blocked vents, dust or a soft surface such as a bed.

Exam points

- Heatsink pulls heat from the chip; the fan removes it from the heatsink.

- Thermal paste fills tiny gaps and improves heat transfer.

- Hot air rises, so intake is at the front and bottom, exhaust at the rear and top.

- 4-pin fans use PWM for better speed control than 3-pin fans.

- AIO liquid coolers are sealed; custom loops need maintenance.

- Thermal throttling reduces speed to protect the chip.

- Positive pressure helps reduce dust.

Day 5: Storage Devices and Power Supply (simple explanation)

Part 1: Storage Devices

What is storage?

Storage keeps your data permanently, even when the power is off. This is the difference from RAM, which is temporary. Your operating system, apps, photos and files all live on storage.

1. HDD (Hard Disk Drive)

- Stores data magnetically on spinning metal platters. A moving read/write head on an arm reads and writes.

- Spindle speed: 5400 or 7200 RPM for desktops and laptops, 10,000 or 15,000 RPM in older servers. Higher RPM is faster.

- Sizes: 3.5-inch (desktop) and 2.5-inch (laptop).

- Speed: roughly 100-200 MB/s.

- Pros: cheap per GB, large capacities. Cons: slow, noisy, easily damaged by shocks, has moving parts.

2. SSD (Solid State Drive)

- Stores data in NAND flash memory chips with no moving parts.

- Speed: SATA SSD about 550 MB/s, NVMe Gen3 about 3,500 MB/s, Gen4 about 7,000 MB/s, Gen5 12,000 MB/s or more.

- Pros: very fast, silent, shock resistant, low power. Cons: costs more per GB, limited write endurance.

- NAND types: SLC (1 bit per cell, fastest and most durable), MLC (2 bits), TLC (3 bits, most common), QLC (4 bits, cheapest, least durable).

- Endurance is rated in TBW (terabytes written).

- TRIM: an OS command that helps an SSD keep its speed. Never defragment an SSD.

3. SSD form factors and interfaces

| Type | Interface | Notes |
| --- | --- | --- |
| 2.5-inch SATA SSD | SATA III (6 Gbps) | Same shape as a laptop HDD |
| M.2 SATA | SATA | Small stick, SATA speed |
| M.2 NVMe | PCIe | Fastest, small stick |
| PCIe add-in card | PCIe | Used in workstations and servers |
| U.2 | PCIe | Enterprise 2.5-inch NVMe |

M.2 sizes are named by width and length, such as 2280 (22 mm x 80 mm), the most common. Key notches (B, M, B+M) must match the slot. Be careful: an M.2 slot may support only SATA, only NVMe, or both.

4. SATA versions

- SATA I: 1.5 Gbps

- SATA II: 3 Gbps

- SATA III: 6 Gbps (about 600 MB/s usable)

The versions are backward compatible. eSATA is the external version. SAS (Serial Attached SCSI) is the server-grade interface.

5. Other storage

- Hybrid drive (SSHD): an HDD with a small SSD cache.

- Optical drives: CD (700 MB), DVD (4.7 GB single layer), Blu-ray (25 GB single layer). Declining in use.

- Flash storage: USB flash drives and memory cards (SD, microSD, CompactFlash).

- Magnetic tape: used for large backups and archives.

- NAS (Network Attached Storage): a storage device shared over the network.

- Cloud storage: data stored on remote servers (Google Drive, OneDrive).

6. RAID (combining drives)

| Level | Name | What it does | Minimum drives | Fault tolerance |
| --- | --- | --- | --- | --- |
| RAID 0 | Striping | Splits data across drives for speed | 2 | None, one failure loses everything |
| RAID 1 | Mirroring | Copies data to two drives | 2 | One drive can fail |
| RAID 5 | Striping with parity | Speed and one-drive protection | 3 | One drive can fail |
| RAID 6 | Double parity | Protection for two failed drives | 4 | Two drives can fail |
| RAID 10 | Mirror plus stripe | Speed and protection | 4 | One per mirrored pair |

RAID is not a backup. It protects against drive failure, not against deleted files or ransomware.

7. Software side of storage

- Partition styles: MBR (up to 2 TB, 4 primary partitions) and GPT (larger disks, UEFI, many partitions).

- File systems: NTFS (Windows), FAT32 (older, 4 GB file limit), exFAT (USB and cards, no 4 GB limit), ext4 (Linux), APFS (Apple).

- Formatting: quick format clears the file table; full format also checks for bad sectors.

- S.M.A.R.T.: drives report health data that tools like CrystalDiskInfo can read.

- Defragmentation: rearranges files on an HDD for speed. Not for SSDs.

Useful tools:

- Windows: Disk Management (diskmgmt.msc), diskpart, chkdsk.

- Linux: lsblk, df -h, fdisk -l.

8. Common storage problems

- Clicking noise from an HDD: failing drive. Back up immediately.

- Drive not detected: loose cable, wrong BIOS setting or an M.2 slot sharing lanes with SATA ports.

- Slow system: nearly full drive, HDD instead of SSD, or a failing disk.

- Bad sectors or file errors: run chkdsk and check S.M.A.R.T. data.

- New disk not showing in File Explorer: it needs to be initialised, partitioned and formatted.

Part 2: Power Supply (PSU)

What does a PSU do?

The power supply unit changes the AC (alternating current) from the wall socket into the low-voltage DC (direct current) the components need. It also protects the PC from power problems.

Voltages (rails)

- +3.3 V and +5 V: motherboard logic, drives and USB.

- +12 V: the most important rail, used by the CPU, GPU and fans.

- Negative voltages (-12 V) are obsolete.

Wattage

Wattage is how much power the PSU can deliver. A gaming PC may need 550-850 W, an office PC 300-450 W. Choose based on your CPU and GPU, plus some headroom, using an online wattage calculator. Quality matters as much as wattage.

Efficiency: 80 PLUS ratings

The rating shows how efficiently the PSU converts AC to DC, so less energy is wasted as heat.

- 80 PLUS (White), Bronze, Silver, Gold, Platinum, Titanium (best).

- Gold is a popular balance of price and efficiency.

Form factors

- ATX: standard desktop PSU.

- SFX / SFX-L: small PSUs for compact cases.

- TFX, Flex ATX: slim cases and OEM PCs.

- The size and mounting must match the case.

Modularity

- Non-modular: all cables are permanently attached.

- Semi-modular: main cables fixed, others detachable.

- Fully modular: all cables detachable, so there is less clutter and better airflow.

Connectors

| Connector | Used for |
| --- | --- |
| 24-pin (20+4) ATX | Main motherboard power |
| 4-pin / 8-pin (4+4) EPS | CPU power |
| 6-pin / 8-pin (6+2) PCIe | Graphics cards |
| 12VHPWR / 12V-2x6 (16-pin) | Newer high-power graphics cards |
| SATA power (15-pin) | SATA drives and some fans |
| Molex (4-pin) | Older drives and fans |
| Floppy (Berg) | Obsolete |

Connectors are keyed, so they only fit one way. Do not force them.

Protection features

- OVP/UVP: over- and under-voltage protection.

- OCP: over-current protection.

- OPP: over-power protection.

- SCP: short-circuit protection.

- OTP: over-temperature protection.

Input voltage

Some PSUs have a 115 V / 230 V switch. It must match your country’s supply. India uses 230 V, so make sure the switch is on 230 V if your PSU has one. Many modern PSUs switch automatically (active PFC).

Power Good signal

When the PSU is ready, it sends a Power Good signal to the motherboard so the CPU can start. If it is missing, the PC will not start.

Safety

- Never open a PSU. Capacitors can hold a dangerous charge even when unplugged.

- Do not use a damaged power cable.

- Use a surge protector or a UPS (Uninterruptible Power Supply) to protect against spikes and power cuts.

Testing a PSU

- PSU tester: plug in the connectors and read the display.

- Multimeter: measure voltages on the connector pins (+12 V should be within about 5%).

- Paperclip test: with the PSU unplugged from the board, connect the green wire (PS_ON) to a black wire (ground) on the 24-pin connector. If the fan spins, the PSU switches on. This does not prove it’s healthy under load.

Common PSU problems

- PC will not turn on: dead PSU, wrong switch position, loose 24-pin, or a faulty power cable.

- Random shutdowns or restarts: overloaded or failing PSU.

- Burning smell, smoke or loud buzzing: switch off and unplug immediately.

- Fan noise or no fan spin: dust or a failed fan.

- Voltage out of range: replace the PSU.

Exam points

- HDD is magnetic with moving parts; SSD is flash with no moving parts.

- NVMe over PCIe is much faster than SATA.

- SATA III is 6 Gbps.

- M.2 2280 means 22 mm wide, 80 mm long.

- RAID 0 = speed, RAID 1 = mirror, RAID 5 = parity, RAID 10 = mirror plus stripe.

- Never defragment an SSD.

- The 24-pin connector powers the motherboard, the 4/8-pin EPS powers the CPU.

- The +12 V rail is the most important.

- 80 PLUS shows efficiency, not quality or wattage.

- Never open a PSU.

Day 6: Expansion Cards, Connectors and Cables (simple explanation)

Part 1: Expansion Cards

What is an expansion card?

An expansion card is a circuit board that plugs into a motherboard slot to add or improve a feature the board doesn’t have or does poorly. Examples are better graphics, extra USB ports, Wi-Fi or professional audio.

Common types

| Card | Purpose |
| --- | --- |
| Video card (GPU) | Better graphics for gaming, video editing and 3D work |
| Sound card | Better audio quality, extra inputs and outputs for music production |
| Network card (NIC) | Wired Ethernet, or Wi-Fi and Bluetooth |
| Capture card | Records or streams video from a console or camera |
| USB / Thunderbolt card | Adds extra or faster ports |
| Storage controller / RAID card | Adds SATA or SAS ports and hardware RAID |
| NVMe adapter card | Lets an M.2 SSD use a PCIe slot |
| Legacy cards | Serial, parallel, modem and TV tuner cards |

Many motherboards now include audio, LAN and Wi-Fi onboard, so cards are needed only for upgrades or special tasks.

Expansion slot types

- PCI: old 32-bit slot, about 133 MB/s. Obsolete.

- AGP: old graphics-only slot. Obsolete.

- PCIe (PCI Express): the current standard. It uses serial lanes, and slot sizes are x1, x4, x8 and x16.

- Mini PCIe / M.2: small slots in laptops and compact boards, often used for Wi-Fi cards and SSDs.

PCIe cards fit in larger slots, so an x1 card works in an x16 slot. A longer card does not fit a shorter slot unless the slot is open-ended.

Approximate bandwidth per lane, per direction:

| PCIe version | Per lane |
| --- | --- |
| 1.0 | 250 MB/s |
| 2.0 | 500 MB/s |
| 3.0 | about 1 GB/s |
| 4.0 | about 2 GB/s |
| 5.0 | about 4 GB/s |

So a PCIe 4.0 x16 slot gives roughly 32 GB/s each way. The PCIe versions are backward compatible, but the link runs at the lower of the card’s and slot’s version.

Physical points:

- Full-height vs low-profile brackets must match the case.

- Riser cards let cards sit sideways in slim cases.

- Many slots are x16 in size but wired as x8 or x4. Check the manual for the real lane count.

- Using some M.2 or PCIe slots may disable certain SATA ports because they share lanes.

Installing an expansion card

- Shut down, unplug the PC and use an ESD strap.

- Open the case and remove the metal slot cover at the back.

- Align the card with the slot and press down firmly until it clicks into the retention clip.

- Fix the bracket to the case with a screw.

- Connect any extra power cable (6-pin, 8-pin or 12VHPWR for graphics cards) and any internal cables the card needs.

- Close the case, reconnect and power on.

- Install the driver from the manufacturer’s website (or let Windows find it).

Configuring a card

- Driver install: the OS needs a driver to talk to the card.

- Device Manager: check that the card appears without a yellow exclamation mark (missing or bad driver).

- BIOS/UEFI: check the card is detected. You may need to set the primary display (onboard or PCIe), or enable features such as Above 4G Decoding or Resizable BAR for modern GPUs.

- Updates: keep drivers and firmware current, using the vendor tool or website.

- Legacy resources: very old systems used IRQ, I/O address and DMA settings. Modern Plug and Play assigns these automatically.

Common card problems

- Card not detected: badly seated, wrong slot, or disabled in BIOS.

- Yellow exclamation in Device Manager: missing or incorrect driver.

- Graphics card shows no display: monitor connected to the motherboard port instead of the card, or the power cable is not connected.

- PC won’t boot after install: card not fully seated or a short from a loose screw.

- Low performance: card running in a slower slot (x4 instead of x16) or an old driver.

- Overheating: poor case airflow or dust on the card’s fans.

Part 2: Connectors and Cables

USB (Universal Serial Bus)

Connector shapes:

- Type-A: the flat rectangular plug on the host side.

- Type-B: square, used for printers.

- Mini-B and Micro-B: small, for older phones and cameras.

- Type-C: small, oval and reversible, used on modern phones, laptops and drives.

Speeds:

| Standard | Speed |
| --- | --- |
| USB 1.1 | 12 Mbps |
| USB 2.0 | 480 Mbps |
| USB 3.0 / 3.2 Gen 1 | 5 Gbps |
| USB 3.1 / 3.2 Gen 2 | 10 Gbps |
| USB 3.2 Gen 2x2 | 20 Gbps |
| USB4 | 40 Gbps |

USB 3.x ports are often blue inside. USB can also deliver power, and USB-C can carry Power Delivery (PD) for laptop charging. USB-C is only the shape: a USB-C port may be slow USB 2.0 or fast USB4, so check the specifications.

Thunderbolt

- Thunderbolt 3 and 4: use the USB-C connector, 40 Gbps, and can carry data, video and power in one cable. Thunderbolt ports also support USB.

- Thunderbolt 1 and 2 used the Mini DisplayPort connector.

Apple Lightning

An 8-pin, reversible connector for older iPhones and iPads. Newer iPhones use USB-C.

Older connectors

- PS/2: round 6-pin. Purple for keyboard, green for mouse.

- Serial (RS-232, DB-9): old modems and network gear console ports.

- Parallel (DB-25): old printers.

- FireWire (IEEE 1394): 400 and 800 Mbps, used for cameras and audio.

Video connectors

| Connector | Type | Notes |
| --- | --- | --- |
| VGA (DB-15) | Analog | Blue, 15 pins, video only, older monitors and projectors |
| DVI | Digital (and analog in some types) | DVI-D digital only, DVI-A analog only, DVI-I both. Single-link or dual-link |
| HDMI | Digital | Video and audio in one cable. Types: A (standard), C (mini), D (micro) |
| DisplayPort (DP) | Digital | Video and audio, supports multiple monitors, Mini DisplayPort variant |
| USB-C / Thunderbolt | Digital | Video through DisplayPort Alt Mode |

HDMI and DisplayPort carry audio; VGA and DVI do not. Day 10 will cover video standards in more depth.

Audio connectors

- 3.5 mm (1/8 inch) jacks, colour coded:

  - Green: line out (speakers and headphones)

  - Pink: microphone

  - Blue: line in

  - Surround setups add orange, black and grey.

- S/PDIF: digital audio, using optical (Toslink) or coaxial connections.

- 1/4 inch (6.35 mm): professional and musical gear.

Networking connectors and cables

- RJ-45: 8-pin plug for Ethernet cables.

- RJ-11: 6-pin (usually 4 wires) plug for phone lines and old modems. It is smaller than RJ-45.

- Coaxial (F-type): used for cable TV and cable internet, with RG-6 or RG-59 cable.

- Fibre optic: connectors SC (square), LC (small), ST (round bayonet). It uses light, and is not affected by electromagnetic interference.

Twisted-pair Ethernet categories:

| Category | Speed (typical) | Max length |
| --- | --- | --- |
| Cat 5 | 100 Mbps | 100 m |
| Cat 5e | 1 Gbps | 100 m |
| Cat 6 | 1 Gbps (10 Gbps up to 55 m) | 100 m |
| Cat 6a | 10 Gbps | 100 m |
| Cat 7 | 10 Gbps | 100 m |
| Cat 8 | 25/40 Gbps | 30 m |

- UTP (unshielded) is common; STP (shielded) reduces interference in noisy places.

- Straight-through cables connect different devices (PC to switch). Crossover cables connect similar devices (PC to PC), though modern ports usually auto-detect.

- Wiring standards: T568A and T568B. Both ends the same = straight-through; one end A and the other B = crossover.

- Plenum-rated cable has a fire-resistant jacket for use in ceilings and ducts.

Day 15 will return to cables and connectors in the networking context.

Internal connectors

- SATA data: thin 7-pin cable, L-shaped.

- SATA power: 15-pin.

- Molex: 4-pin power, older.

- Front-panel, USB and audio headers on the motherboard (see Day 1).

Software side

- Device Manager: shows all devices, driver status and errors.

- Plug and Play: the OS and firmware detect and configure new devices automatically.

- Drivers: needed for cards, USB devices and other peripherals.

- Windows commands: devmgmt.msc for Device Manager.

- Linux commands: lspci lists PCIe devices, lsusb lists USB devices.

- Safely remove hardware for USB storage, to avoid corrupting data.

Common connector and cable problems

- No display: wrong input on the monitor, cable loose, or the cable plugged into the motherboard port instead of the card.

- USB device slow: plugged into a USB 2.0 port or using a USB 2.0 cable.

- Ethernet not working: cable damaged, wrong category for the speed, or length over 100 m.

- No sound: plugged into the wrong coloured jack, muted, or the wrong output device selected.

- Bent pins on VGA or DVI connectors cause missing colours or a blurry picture.

Exam points

- PCIe lanes are x1, x4, x8, x16 and the slots are backward compatible.

- PCIe is the current expansion slot; PCI and AGP are obsolete.

- A yellow exclamation in Device Manager means a driver problem.

- USB 2.0 is 480 Mbps, 3.x is 5 Gbps or more, and USB-C is only a connector shape.

- Thunderbolt 3/4 use the USB-C connector at 40 Gbps.

- VGA is analog; DVI can be digital or analog; HDMI and DisplayPort carry digital video and audio.

- Green is line out, pink is mic, blue is line in.

- PS/2 purple is the keyboard, green is the mouse.

- Cat 5e supports 1 Gbps; Cat 6a supports 10 Gbps at 100 m.

- RJ-45 is Ethernet (8 pins), RJ-11 is phone (smaller).

- Fibre connectors are SC, LC and ST.

Day 7: Input Devices (simple explanation)

What is an input device?

An input device sends data and commands into the computer. It turns human actions (typing, clicking, speaking, scanning) into electrical signals the computer can process. This is the “Input” step of the Input-Process-Output cycle.

Analogy: input devices are the computer’s senses: ears (microphone), eyes (scanner and camera), and hands (keyboard and mouse).

1. Keyboard

Parts and layout

- Alphanumeric keys: letters, numbers and symbols.

- Function keys (F1-F12): shortcuts that depend on the program.

- Modifier keys: Shift, Ctrl, Alt, Windows/Command, used in combinations.

- Navigation keys: arrows, Home, End, Page Up, Page Down.

- Numeric keypad: for fast number entry, controlled by Num Lock.

- Special keys: Esc, Tab, Caps Lock, Backspace, Delete, Print Screen.

- Layouts: QWERTY is the standard. Dvorak and Colemak are alternatives. Others exist for many languages, such as AZERTY (French).

Types of keyboard

| Type | How it works | Notes |
| --- | --- | --- |
| Membrane | Rubber dome presses a circuit sheet | Cheap, quiet, soft feel |
| Scissor-switch | Dome with a scissor mechanism | Used in laptops, slim and stable |
| Mechanical | Individual switch under each key | Durable, precise, louder, popular with gamers and typists |
| Ergonomic / split | Angled or split layout | Reduces wrist strain |
| Virtual / on-screen | Drawn on a touchscreen | Phones, tablets, accessibility |
| Projection | Laser projects keys on a surface | Rare |

Mechanical switch types are often described as linear (smooth), tactile (bump) or clicky (bump and sound). Gaming keyboards often offer N-key rollover (NKRO), so many keys register at once, and anti-ghosting, which prevents missed key presses.

Connection types

- PS/2: purple round connector, older.

- USB: wired standard today.

- Wireless: a 2.4 GHz RF dongle, or Bluetooth.

- Infrared (IR): rare and needs a clear line of sight.

2. Mouse and other pointing devices

Mouse types

- Optical: uses an LED and a sensor to track movement. It is the standard.

- Laser: more sensitive and works on more surfaces, including glossy ones.

- Mechanical (ball): obsolete, needed regular cleaning.

- Trackball: you roll a ball with your thumb or fingers. The device stays still.

- Vertical / ergonomic mouse: upright grip for less wrist strain.

- Gaming mouse: high DPI, programmable buttons and adjustable weights.

Mouse characteristics

- DPI (dots per inch): pointer sensitivity. Higher DPI means a faster pointer for the same hand movement. Typical office mice are 800-1600 DPI.

- Polling rate (Hz): how often the mouse reports its position, such as 125 or 1000 Hz. Higher means smoother response.

- Buttons: left click, right click, scroll wheel (which also acts as a button), and extra side buttons.

- Connection: USB, PS/2 (green), 2.4 GHz wireless, or Bluetooth.

Other pointing devices

- Touchpad / trackpad: a touch-sensitive surface on a laptop. It supports gestures such as two-finger scroll, pinch zoom and three-finger swipe.

- Pointing stick (TrackPoint): a small rubber nub between the keys on some laptops.

- Stylus and graphics tablet (digitizer): a pen on a pressure-sensitive pad, used for drawing and design. It senses pressure levels.

- Touchscreen: you touch the display directly (see below).

3. Touchscreens

| Type | How it works | Notes |
| --- | --- | --- |
| Capacitive | Detects the electrical charge of your finger | Used in phones and tablets, supports multi-touch, needs a finger or special stylus |
| Resistive | Two layers press together | Works with gloves or any stylus, older ATMs and kiosks, less clear |
| Infrared | Beams across the screen are interrupted | Large displays and kiosks |
| Surface acoustic wave (SAW) | Sound waves across glass | Clear picture, used in kiosks |

Multi-touch allows pinch, rotate and swipe gestures. Touchscreens are both input and output devices.

4. Scanners and readers

Document and image scanners

- Flatbed: place a page on glass. Best quality, good for books and photos.

- Sheet-fed: pulls paper through. Fast for many pages, often with an ADF (automatic document feeder).

- Handheld: moved by hand over the item.

- Drum scanner: professional, very high quality.

- Resolution is measured in DPI. 300 DPI is common for documents, 600 DPI or more for photos.

- OCR (Optical Character Recognition): software that turns a scanned image of text into editable text.

- Colour depth: how many colours are captured, such as 24-bit.

- Scanners use CCD or CIS sensors. CCD gives higher quality; CIS is thinner and cheaper.

Barcode and QR readers

- Laser scanner: reads 1D barcodes with a moving laser line.

- CCD / linear imager: reads 1D barcodes with a sensor.

- 2D imager / camera: reads both barcodes and QR codes.

- Used in shops, warehouses and libraries. They usually connect by USB, Bluetooth or wirelessly, and often act like a keyboard that “types” the code.

Card and tag readers

- Magnetic stripe reader: swipe a card, used for older bank cards.

- Smart card reader: reads a chip card, used for secure logins and ID.

- RFID reader: reads radio tags, used in access cards and inventory.

- NFC: short-range (a few centimetres) wireless used for contactless payments and pairing.

5. Audio and video input

- Microphone: converts sound to a signal. Types include dynamic (rugged) and condenser (sensitive, often needs power, called phantom power). Connect by 3.5 mm pink jack, USB or XLR.

- Webcam: captures video for calls and streaming. Resolution examples are 720p, 1080p and 4K. Most connect by USB, or are built into laptops.

- Digital camera and camcorder: connect by USB, or through memory cards.

- Capture card: takes video from consoles or cameras.

- MIDI devices: music keyboards and controllers that send note and control data, not actual sound.

6. Biometric devices

These identify you by body characteristics:

- Fingerprint reader: very common on laptops and phones.

- Facial recognition: for example Windows Hello, which uses an infrared camera.

- Iris and retina scanners: very accurate, used in high security.

- Voice recognition.

Biometrics are convenient and hard to share, but they are best combined with a PIN or password (two-factor authentication).

7. Gaming and other input

- Gamepad / controller, joystick, steering wheel and pedals, flight sticks.

- VR headset and controllers: track head and hand movement.

- Motion sensors: accelerometer and gyroscope in phones and controllers.

- KVM switch (Keyboard, Video, Mouse): lets one keyboard, monitor and mouse control several computers.

8. Software side

- Drivers and HID: many keyboards and mice are HID (Human Interface Device) class, so Windows uses a built-in driver automatically. Special features (macros, RGB lighting) need the vendor’s software.

- Device Manager: shows keyboards, mice and other devices, and their status.

- Mouse settings (Windows Settings > Bluetooth & devices > Mouse): pointer speed, button swapping, scroll direction, double-click speed.

- Keyboard settings: language, layout, repeat delay and repeat rate.

- Accessibility features:

  - Sticky Keys: press Shift five times; lets you press key combinations one key at a time.

  - Filter Keys: hold the right Shift key for 8 seconds; ignores brief or repeated key presses.

  - Toggle Keys: hold Num Lock for 5 seconds; plays a sound when Caps/Num Lock changes.

  - On-Screen Keyboard, Narrator, magnifier and speech recognition / voice typing.

- Bluetooth pairing: turn on Bluetooth, put the device in pairing mode, select it from the list, and confirm any code.

- Touchscreen calibration: adjusts accuracy in Windows settings.

- Scanner software: uses TWAIN or WIA (Windows Image Acquisition) drivers so apps can scan.

- Windows Hello setup: Settings > Accounts > Sign-in options.

9. Common problems

- Keyboard or mouse not working: check the cable or USB port, try another port, replace batteries or recharge, re-pair Bluetooth, and check Device Manager.

- Wireless lag or dropouts: low battery, interference (Wi-Fi, other 2.4 GHz devices), distance, or a USB 3.0 port causing interference for the dongle. Try a different port or move the dongle closer.

- Erratic pointer: dirty sensor, unsuitable surface (glass), or a dirty mouse pad. Clean the sensor and use a mouse pad.

- Wrong characters typed: wrong keyboard layout or language selected.

- Sticky or non-working keys: dirt or spilled liquid. Clean carefully, or replace the keyboard.

- Touchscreen unresponsive or inaccurate: dirty screen, needs calibration, or a driver problem.

- Scanner not recognised: driver missing, cable loose, or the scanner lock not released (many flatbeds have a transport lock).

- Scans are blurry or streaked: dirty glass or the wrong resolution setting.

- Fingerprint reader failing: dirty or wet finger, or the print needs to be re-enrolled.

- Microphone not working: wrong input device selected, muted, or the privacy setting blocks it.

Exam points

- Input devices send data to the computer, and output devices receive it.

- PS/2 purple is the keyboard, green is the mouse.

- Mechanical keyboards use a switch under each key; membrane keyboards use rubber domes.

- DPI measures mouse sensitivity and scanner resolution.

- Capacitive touchscreens need a finger; resistive screens work with any stylus or glove.

- OCR turns scanned text into editable text.

- TWAIN and WIA are scanner driver standards.

- Sticky Keys is activated by pressing Shift five times.

- Biometrics are fingerprint, face, iris and voice.

- A KVM switch shares one keyboard, video and mouse between several computers.

- NFC is very short range, and RFID uses radio tags.

- Bluetooth needs pairing; 2.4 GHz wireless uses a USB dongle.

Day 8: Output Devices (simple explanation)

What is an output device?

An output device takes the results of processing from the computer and presents them to the user in a form we can see, hear or hold. This is the “Output” step of the Input-Process-Output cycle.

Analogy: if input devices are the computer’s senses, output devices are its mouth and hands: it speaks through speakers, shows through monitors and writes through printers.

Output can be soft copy (temporary, such as the screen or sound) or hard copy (permanent, such as printed paper).

1. Monitors and displays (overview)

The monitor is the main output device. Day 9 covers display types and settings in detail, and Day 10 covers video standards, so this is the short version.

- LCD (Liquid Crystal Display): a backlight shines through liquid crystals. Panel types are TN (fast, weak colours), IPS (best colours and viewing angles) and VA (good contrast).

- LED monitor: an LCD that uses LED backlighting. This is what most “LED” monitors are.

- OLED: each pixel makes its own light, so blacks are truly black and contrast is excellent. Risk of burn-in on static images.

- CRT: old, bulky tube monitors. Obsolete.

- Touchscreen displays: input and output in one.

Key terms:

- Resolution: number of pixels, such as 1920x1080 (Full HD) or 3840x2160 (4K).

- Refresh rate (Hz): how many times per second the picture updates. 60 Hz is standard, 144 Hz or more for gaming.

- Response time (ms): how fast a pixel changes colour. Lower means less blur.

- Aspect ratio: width to height, usually 16:9, or 21:9 for ultrawide.

2. Projectors

A projector throws a large image onto a screen or wall. It is used in classrooms, meetings and home theatres.

| Type | How it works | Notes |
| --- | --- | --- |
| DLP | Tiny mirrors on a chip reflect light | Sharp, compact, can show a “rainbow effect” for some viewers |
| LCD | Light passes through LCD panels | Good colours, brighter colours |
| LCoS | Reflective liquid crystal on silicon | High quality, expensive |
| Laser / LED | Light source is a laser or LED | Long life, instant on |

Key terms:

- Lumens: brightness. Around 2,000-3,500 lumens suits a room with some light, and more for large rooms.

- Contrast ratio: difference between the brightest white and darkest black.

- Native resolution: the real pixel count. Match it to your source.

- Throw distance: how far the projector must be from the screen. Short-throw projectors work close to the wall.

- Keystone correction: fixes a trapezoid-shaped picture when the projector sits at an angle.

- Lamp life: traditional lamps last a few thousand hours and get hot. Let the fan cool the projector before unplugging it.

3. Printers (overview)

Printers create hard copies. Days 23-25 cover them in depth, so here are just the main types.

| Type | How it works | Best for |
| --- | --- | --- |
| Inkjet | Sprays tiny ink droplets | Photos, home use |
| Laser | Uses toner, a drum and heat (fuser) | Fast, sharp text, offices |
| Thermal | Heats special paper | Receipts, shipping labels |
| Impact (dot matrix) | Pins strike a ribbon | Multipart carbon forms, invoices |
| 3D printer | Builds objects layer by layer | Prototypes, parts |
| Plotter | Pens or ink for large drawings | Architecture, engineering |

Key terms:

- DPI: print resolution.

- PPM: pages per minute, the speed.

- Duplex: printing on both sides.

- MFP / MFD (multifunction printer): print, scan, copy and sometimes fax.

4. Audio output devices

Speakers

- Stereo (2.0): two speakers, left and right.

- 2.1: two speakers plus a subwoofer for deep bass.

- 5.1 / 7.1 surround: several speakers and a subwoofer for movies and games. The “.1” is the subwoofer.

- Soundbar: one long speaker bar, common with TVs.

- Active vs passive speakers: active speakers have a built-in amplifier and need power; passive speakers need an external amplifier.

- Wireless speakers: connect by Bluetooth or Wi-Fi.

Headphones and headsets

- Over-ear, on-ear and in-ear (earbuds) designs.

- Open-back (natural sound, leaks noise) vs closed-back (blocks noise, isolates).

- Headset: headphones with a microphone.

- Noise cancelling: microphones pick up outside noise and the headphones cancel it.

- Connections: 3.5 mm jack, USB, USB-C, Bluetooth.

Sound characteristics

- Frequency response: the range of pitches a device can reproduce, usually 20 Hz to 20,000 Hz for human hearing.

- Impedance (ohms): electrical resistance. High-impedance headphones need an amplifier.

- Sensitivity (dB): how loud it plays for a given power.

- Sampling rate and bit depth: sound quality of digital audio, for example 44.1 kHz and 16-bit for CDs, or 48 kHz and 24-bit for video and studio audio.

Audio connectors (from Day 6)

- Green 3.5 mm jack is line out.

- S/PDIF (optical or coaxial) and HDMI carry digital audio.

- Bluetooth and USB are common for wireless and digital audio.

5. Other output devices

- Braille display: raises pins to show text for blind users.

- Speech synthesizer: reads text aloud.

- Haptic feedback: vibration in phones and game controllers.

- VR headsets: display and sound output.

- LED indicators, LCD displays in printers, routers and appliances.

- Plotters, label printers and signage for specialised uses.

- Data projectors and interactive whiteboards in education.

6. Output devices vs input devices vs both

- Input only: keyboard, mouse, scanner, microphone.

- Output only: monitor (non-touch), printer, speakers, projector.

- Both input and output: touchscreen, headset (speakers and mic), multifunction printer (print and scan), network card, storage drives.

7. Software side

- Drivers: printers, sound cards and graphics cards need drivers. Windows may install generic drivers, but the vendor’s driver gives full features.

- Sound settings (Settings > System > Sound): choose the default output device, set volume, and test speakers. Other tools include the Volume Mixer for per-app volume.

- Sound control panel (mmsys.cpl): playback devices, enhancements, spatial sound and the sample format.

- Display settings (Settings > System > Display): resolution, scale, orientation, refresh rate and multiple monitors. You can duplicate or extend displays. The shortcut Windows + P switches projection mode, which is useful with projectors.

- Printers and scanners settings: add printers, set the default printer, view the print queue, and run the troubleshooter.

- Print spooler: the Windows service that holds print jobs in the queue. If a job gets stuck, restarting the Print Spooler service often fixes it.

- Bluetooth audio: pair the speaker or headset, then select it as the output device. Latency can be noticeable for games and video.

- Audio drivers and codecs: the codec decides how sound is compressed and played. Examples include MP3, AAC, FLAC and Dolby.

8. Common problems

- No picture on the monitor: check power, cable and the monitor’s input source, then try another cable or port.

- Wrong resolution or blurry picture: set the monitor’s native resolution and update the graphics driver.

- Flickering screen: loose cable, wrong refresh rate or a faulty driver.

- Dead pixels or burn-in: damaged pixels or a ghost image left by static content (OLED). Dead pixels usually cannot be repaired.

- Projector image dim or yellow: the lamp is ageing, filters are dirty, or ambient light is too high.

- Projector shows “No signal”: wrong input source, or the laptop is not set to output to the projector (press Windows + P).

- Trapezoid-shaped picture: adjust keystone correction or reposition the projector.

- No sound: muted, wrong output device selected, volume turned down, cable in the wrong jack, or a driver problem.

- Crackling or distorted sound: bad cable, interference, volume too high or a failing speaker.

- Bluetooth audio lag or dropouts: distance, interference or low battery.

- Printer problems: (covered in Days 23-25) such as paper jams, streaks, and jobs stuck in the queue.

Exam points

- Output devices present results to the user; examples are monitor, printer, speakers and projector.

- Soft copy is on-screen or sound, and hard copy is printed.

- Touchscreens, headsets and multifunction printers are both input and output devices.

- Resolution, refresh rate and response time describe a monitor.

- Projector brightness is measured in lumens, and keystone fixes a trapezoid image.

- DLP uses mirrors; LCD projectors use liquid crystal panels.

- Inkjet sprays ink, laser uses toner and a fuser, thermal heats paper, and impact printers strike a ribbon.

- 5.1 audio means five speakers plus one subwoofer.

- Windows + P changes how the display is projected.

- Restarting the Print Spooler can fix a stuck print queue.

- Active speakers have a built-in amplifier.

Day 9: Display Types and Settings (simple explanation)

What is a display?

A display is the main output device that shows the computer’s visual output. It is made of millions of tiny dots called pixels, and each pixel mixes red, green and blue (RGB) light to make any colour.

1. Display technologies

CRT (Cathode Ray Tube)

- Old, heavy monitors that fire an electron beam at a phosphor-coated screen.

- Obsolete, but they still appear in exam questions. They contain dangerous high voltage and lead, so they need special recycling.

LCD (Liquid Crystal Display)

- Liquid crystals twist to let light through or block it. They don’t make their own light, so a backlight is needed.

- Panel types:

| Panel | Strengths | Weaknesses |
| --- | --- | --- |
| TN (Twisted Nematic) | Fastest response, cheapest | Poor colours and viewing angles |
| IPS (In-Plane Switching) | Best colour accuracy, wide viewing angles | Costs more, weaker blacks, slower than TN in the past |
| VA (Vertical Alignment) | Best contrast and deep blacks | Slower response, medium viewing angles |

LED displays

- An LED monitor is really an LCD with LED backlighting, not a different panel. LEDs replaced the older CCFL backlight tubes.

- Edge-lit: LEDs along the edges, thinner and cheaper.

- Direct-lit / full-array: LEDs behind the whole panel, with better brightness uniformity. Local dimming darkens zones for better blacks.

- Mini-LED: thousands of tiny LEDs for finer local dimming and higher brightness.

OLED (Organic LED)

- Each pixel makes its own light, so no backlight is needed.

- Pros: perfect blacks, excellent contrast, very fast response, thin and flexible.

- Cons: costs more, risk of burn-in (a ghost image from static content like taskbars), lower sustained brightness in some models.

- AMOLED is common in phones.

QLED

- A Samsung term for an LCD with a quantum-dot layer, which improves colour and brightness. It is not an OLED.

Plasma

- Obsolete. Small cells of gas glow when charged. It was heavy and power hungry.

Projectors

- Covered in Day 8: DLP, LCD and LCoS types.

2. Key display characteristics

| Term | Meaning |
| --- | --- |
| Resolution | Number of pixels (width x height). More pixels give a sharper picture. |
| Native resolution | The panel’s real resolution. Always use it for the sharpest image. |
| Aspect ratio | Width to height shape: 4:3 (old), 16:9 (standard widescreen), 16:10, 21:9 (ultrawide). |
| Pixel density (PPI) | Pixels per inch. Higher means a sharper image. |
| Refresh rate (Hz) | How many times per second the screen redraws. 60 Hz is normal, 120-360 Hz for gaming. |
| Response time (ms) | How fast a pixel changes colour. Lower reduces ghosting and motion blur. |
| Brightness (nits / cd/m2) | Light output. 250-350 nits is normal indoors; HDR needs 600 or more. |
| Contrast ratio | Difference between white and black, for example 1000:1. |
| Colour depth | Bits per colour. 8-bit gives 16.7 million colours; 10-bit gives over a billion. |
| Colour gamut | The range of colours shown: sRGB (standard), Adobe RGB (print work), DCI-P3 (video and cinema). |
| Viewing angle | The angle at which the picture still looks right. IPS is best. |
| Curved display | Slight curve for immersion, mostly ultrawide and gaming monitors. |

Common resolutions

| Name | Resolution |
| --- | --- |
| VGA | 640 x 480 |
| SVGA | 800 x 600 |
| XGA | 1024 x 768 |
| HD (720p) | 1280 x 720 |
| Full HD (FHD, 1080p) | 1920 x 1080 |
| QHD (WQHD, 1440p) | 2560 x 1440 |
| 4K UHD | 3840 x 2160 |
| 8K UHD | 7680 x 4320 |

Day 10 covers video standards, like these resolutions and HDMI/DisplayPort versions, in more depth.

Other important technologies

- HDR (High Dynamic Range): a wider range between dark and bright, so images look more realistic. It needs a capable panel (HDR10, Dolby Vision).

- Adaptive sync (G-Sync from NVIDIA, FreeSync from AMD, VESA Adaptive-Sync): the monitor matches the graphics card’s frame rate, which removes screen tearing in games.

- V-Sync: a software option that locks frames to the refresh rate. It stops tearing but can add input lag.

- Flicker-free and low blue light modes reduce eye strain.

- VESA mount: the standard hole pattern (75 x 75 or 100 x 100 mm) for wall or arm mounts.

3. Laptop and mobile displays

- Laptop displays are LCD (usually IPS) or OLED, with an LED backlight.

- Parts in a laptop screen assembly: the panel, backlight, digitizer (touch layer on touch models), webcam, microphone, and the Wi-Fi antenna wires that run through the lid.

- Older laptops used a CCFL backlight with an inverter board. Failure of the inverter or backlight gives a very dim picture.

- Connection: an internal cable (LVDS or eDP) links the screen to the motherboard. A damaged cable gives flicker, lines or a blank screen.

- Phones and tablets use IPS LCD or OLED/AMOLED with capacitive touch.

- Auto-brightness uses the ambient light sensor, and auto-rotate uses the accelerometer/gyroscope.

4. Display settings (Windows)

Open Settings > System > Display, or right-click the desktop and choose Display settings. The older Control Panel applet runs from desk.cpl.

- Resolution: choose the recommended (native) one. A non-native resolution looks blurry.

- Scale and layout (DPI scaling): 100%, 125%, 150% and so on, makes text and icons larger on high-resolution screens.

- Orientation: landscape, portrait, flipped.

- Refresh rate: under Advanced display. Pick the highest the monitor supports.

- Multiple displays:

  - Duplicate: same picture on both screens, ideal for presentations.

  - Extend: the desktop is spread across both screens, for more workspace.

  - Second screen only / PC screen only: use only one display.

  - Drag the numbered boxes to match the physical position and set the main display.

  - Shortcut: Windows + P.

- Night light: warmer colours in the evening.

- HDR and colour settings: turn on HDR, adjust colour profile and Calibrate display colour (Display Colour Calibration tool).

- Brightness and contrast: laptops use Settings or Fn keys. External monitors use their on-screen menu (OSD) buttons.

- Graphics driver control panels (NVIDIA Control Panel, AMD Software, Intel Graphics Command Center) offer more advanced options.

- Scale mode: keep aspect ratio or fill. The graphics driver may stretch the image.

- Screen saver and power settings: turn display off after inactivity.

Other platforms

- macOS: System Settings > Displays.

- Linux: Settings > Displays, or the xrandr command in X11.

- dxdiag (Windows) shows display adapter and monitor information.

Cables and connections

HDMI, DisplayPort, DVI, VGA and USB-C carry the video signal (see Day 6). Use a cable and port that support your resolution and refresh rate, for example DisplayPort or HDMI 2.0 or higher for 4K at 60 Hz.

5. Common problems and fixes

| Problem | Likely cause | Fix |
| --- | --- | --- |
| No display | Cable loose, wrong input, no power, or cable plugged into the motherboard instead of the graphics card | Check power, cable and input source; try another cable or port |
| “No signal” or “Input not supported” | Wrong resolution or refresh rate | Boot in Safe Mode / low-resolution mode and set a supported mode |
| Blurry image | Non-native resolution | Set the native resolution |
| Flickering | Loose cable, wrong refresh rate, bad driver, or interference | Reseat or replace the cable; check refresh rate; update the driver |
| Dim image | Brightness low, failing backlight or inverter | Raise brightness; with a flashlight you can see a faint image if the backlight has failed |
| Dead or stuck pixels | Faulty transistors | Pixel-fixing tools sometimes help; otherwise warranty or panel replacement |
| Burn-in / image retention | Static image on OLED, or temporary on LCD | Use screen savers, move windows, hide the taskbar; LCD retention may fade |
| Ghosting or motion blur | Slow response time or a low refresh rate | Enable overdrive in the OSD, raise the refresh rate |
| Colour tint (all red, green or blue) | Loose or damaged cable or pins | Reseat or replace the cable; check for bent pins on VGA/DVI |
| Vertical or horizontal lines | Damaged cable or panel, or graphics fault | Try another cable and monitor; update drivers |
| Distorted or stretched image | Wrong aspect ratio setting | Correct the resolution and scaling |
| Screen tearing in games | Frame rate and refresh rate out of sync | Enable adaptive sync or V-Sync |
| Text too small or too large | Scaling setting | Adjust Scale in Display settings |
| Second monitor not detected | Cable, input source, driver, or not in extended mode | Click Detect in Display settings; check the cable and driver |
| Laptop screen blank but external works | Internal cable, panel, or backlight failure | Try external monitor; inspect the screen cable or replace the panel |
| Screen shows only part of the picture or wrong size | Overscan or scaling in the TV or monitor | Change the picture size or screen-fit setting |

Basic troubleshooting steps

- Check the power and cables, and the monitor’s input source.

- Test with a known-good cable, monitor or port.

- Check Display settings for resolution and refresh rate.

- Update or roll back the graphics driver.

- Try the display on another computer to decide if the monitor or the PC is at fault.

- For a laptop, connect an external monitor to isolate the screen from the graphics system.

Care and safety

- Clean the screen with a soft, dry microfibre cloth (slightly damp if needed). Never spray liquid directly on it and avoid ammonia-based cleaners.

- Power off before cleaning.

- LCD backlights and CRTs can hold dangerous voltage, so do not open them. Dispose of old displays through e-waste recycling because of lead and mercury (in older CCFL backlights).

Exam points

- An LED monitor is an LCD with LED backlighting.

- OLED pixels make their own light, with perfect blacks, but can suffer burn-in.

- IPS: best colour and viewing angles. TN: fastest and cheapest. VA: best contrast.

- Native resolution gives the sharpest picture.

- Full HD = 1920x1080, QHD = 2560x1440, 4K UHD = 3840x2160.

- Refresh rate is in Hz, and response time is in ms.

- FreeSync and G-Sync reduce screen tearing.

- Duplicate mirrors the display; Extend widens the desktop; Windows + P switches modes.

- A very dim image with a faint picture under a flashlight suggests backlight or inverter failure.

- Colour tint often means a bad cable or bent pins.

- Test with a known-good cable or monitor.

- Never spray liquid on the screen.

Day 10: Video Standards and Technology (simple explanation)

What are video standards?

A video standard is an agreed set of rules for how video is created, sent and shown: the resolution, colours, refresh rate, connector type and signal format. Standards let any graphics card, cable and monitor from different makers work together.

1. Evolution of video standards

| Standard | Resolution | Colours | Notes |
| --- | --- | --- | --- |
| MDA | 720 x 350 (text only) | Monochrome | First IBM standard, no graphics |
| CGA | 320 x 200 | 4 | First colour graphics |
| EGA | 640 x 350 | 16 | Better colour |
| VGA | 640 x 480 | 16 (256 at 320 x 200) | Introduced analog video, the base for all later standards |
| SVGA | 800 x 600 | 65,000+ | “Super VGA” |
| XGA | 1024 x 768 | 65,000+ | 4:3 shape |
| SXGA | 1280 x 1024 | Millions | 5:4 shape |
| UXGA | 1600 x 1200 | Millions | 4:3 shape |

Widescreen versions:

| Name | Resolution | Ratio |
| --- | --- | --- |
| WXGA | 1280 x 800 | 16:10 |
| WXGA+ | 1440 x 900 | 16:10 |
| WSXGA+ | 1680 x 1050 | 16:10 |
| WUXGA | 1920 x 1200 | 16:10 |

High-definition and beyond:

| Name | Resolution | Pixels (approx.) |
| --- | --- | --- |
| HD (720p) | 1280 x 720 | 0.9 million |
| Full HD (1080p) | 1920 x 1080 | 2 million |
| QHD / WQHD (1440p) | 2560 x 1440 | 3.7 million |
| 4K UHD (2160p) | 3840 x 2160 | 8.3 million |
| 8K UHD | 7680 x 4320 | 33 million |

4K in cinema (DCI 4K) is 4096 x 2160.

2. Key video terms

- Resolution: how many pixels across and down.

- Aspect ratio: the shape of the picture. 4:3 (old), 16:9 (HD standard), 16:10, 21:9 (ultrawide).

- Refresh rate (Hz): screen redraws per second.

- Frame rate (fps): frames per second in a video. 24 fps is film, 30 fps is standard video, 60 fps is smoother for games and sports.

- Interlaced (i) vs progressive (p):

  - Interlaced draws odd lines, then even lines (480i, 1080i). It saves bandwidth but can show flicker or combing.

  - Progressive draws every line in each frame (720p, 1080p, 4K). It is the standard today.

- Colour depth: bits per colour channel. 8-bit gives 16.7 million colours, 10-bit gives about a billion.

- Chroma subsampling (4:4:4, 4:2:2, 4:2:0): how much colour detail is stored compared to brightness. 4:4:4 is full quality, 4:2:0 is common in streaming and Blu-ray.

- HDR (High Dynamic Range): a wider range between dark and bright. Common formats are HDR10, HDR10+ and Dolby Vision.

- Bitrate: how much data per second a video uses (Mbps). Higher gives better quality and bigger files.

- Analog vs digital signal:

  - Analog (VGA, component, composite) can pick up noise and blur.

  - Digital (DVI-D, HDMI, DisplayPort) is exact and sharper.

- HDCP (High-bandwidth Digital Content Protection): copy protection that makes sure protected video only plays over compliant connections. If one device in the chain doesn’t support it, you may get a black screen or error with streaming or Blu-ray.

3. Video connectors (more detail)

| Connector | Signal | Max (typical) | Audio? | Notes |
| --- | --- | --- | --- | --- |
| VGA (DE-15) | Analog | Up to about 1920 x 1200, quality drops | No | Blue, 15 pins, screws to hold |
| DVI-D | Digital | Single link 1920 x 1200 at 60 Hz; dual link 2560 x 1600 | No |  |
| DVI-A | Analog |  | No | Works with VGA |
| DVI-I | Both |  | No | Integrated, carries analog and digital |
| HDMI | Digital | Depends on version | Yes | Common on TVs, consoles, laptops |
| DisplayPort | Digital | Depends on version | Yes | PC monitors, multi-monitor |
| USB-C (DP Alt Mode) | Digital | Depends on port | Yes | One cable for video, data and power |
| Thunderbolt 3/4 | Digital | 40 Gbps | Yes | Carries DisplayPort video and more |
| Composite | Analog | 480i | No | Yellow RCA, old TVs and VCRs |
| Component (YPbPr) | Analog | Up to 1080i/1080p | No | Three RCA plugs: red, green, blue |
| S-Video | Analog | 480i | No | 4-pin mini-DIN, old |

HDMI versions

| Version | Bandwidth | Maximum video (typical) | Notes |
| --- | --- | --- | --- |
| 1.4 | 10.2 Gbps | 4K at 30 Hz, 1080p at 120 Hz | Added ARC (audio return) |
| 2.0 | 18 Gbps | 4K at 60 Hz | HDR support |
| 2.1 | 48 Gbps | 4K at 120 Hz, 8K at 60 Hz | VRR, eARC, dynamic HDR |

- Connector types: Type A (standard, 19 pins), Type C (mini), Type D (micro).

- HDMI Ethernet, ARC and eARC let the TV send audio back to a soundbar.

- Use an Ultra High Speed cable for HDMI 2.1.

DisplayPort versions

| Version | Bandwidth | Typical maximum video |
| --- | --- | --- |
| 1.2 | 21.6 Gbps | 4K at 60 Hz |
| 1.4 | 32.4 Gbps | 4K at 120 Hz, 8K at 60 Hz with compression (DSC) |
| 2.0 / 2.1 | up to 80 Gbps | 8K and very high refresh rates |

- Locking connector with a latch, and a Mini DisplayPort version.

- MST (Multi-Stream Transport) allows daisy-chaining several monitors from one port.

- DisplayPort is royalty-free, while HDMI is licensed.

Adapters

- A passive adapter only converts the connector (for example DisplayPort to HDMI when the port supports dual-mode).

- An active adapter converts the signal (digital to analog, such as HDMI to VGA).

- You cannot reliably go from VGA (analog) to HDMI or DisplayPort with a simple cable; an active converter is needed.

- Adapters and long cables can limit resolution or refresh rate.

4. Video cards (GPUs)

A GPU (Graphics Processing Unit) is a processor specialised for graphics and parallel calculations.

Integrated vs dedicated

|  | Integrated GPU | Dedicated GPU |
| --- | --- | --- |
| Location | Built into the CPU or chipset | Separate card (PCIe) |
| Memory | Shares system RAM | Own VRAM |
| Power and heat | Low | High, needs a bigger cooler and often extra power cables |
| Performance | Office, video playback, light games | Gaming, 3D, video editing, AI work |
| Examples | Intel UHD/Iris Xe, AMD Radeon Graphics | NVIDIA GeForce, AMD Radeon RX, Intel Arc |

Key GPU specifications

- VRAM (video memory): type and size, such as 8 GB GDDR6. Higher resolutions and textures need more.

- Memory bus and bandwidth: how quickly data moves.

- GPU cores: CUDA cores (NVIDIA), stream processors (AMD).

- Clock speed (base and boost).

- TDP and power connectors: 6-pin, 8-pin, or 12VHPWR.

- Outputs: HDMI, DisplayPort ports on the card.

- Slot width: many cards are 2-3 slots thick, so check case clearance.

- Multi-GPU: SLI (NVIDIA) and CrossFire (AMD) linked two cards, but they are now rare. Modern systems usually use one powerful GPU.

- Ray tracing and upscaling: features such as DLSS, FSR and XeSS render at lower resolution and upscale, improving frame rates.

Graphics APIs

APIs let software talk to the GPU:

- DirectX (Windows). DirectX 12 is current. Check with dxdiag.

- OpenGL (cross-platform) and Vulkan (modern, cross-platform).

- Metal (Apple).

5. Video formats, codecs and containers

- Codec (coder-decoder): compresses and decompresses video. Common ones:

  - H.264 / AVC: very widely supported.

  - H.265 / HEVC: about half the file size at the same quality, needs more processing.

  - VP9 and AV1: royalty-free, AV1 is the newest and most efficient.

  - MPEG-2: used for DVDs and older broadcast.

- Container: the file format that holds video, audio and subtitles. Examples: MP4, MKV, AVI, MOV, WebM.

- Audio codecs: AAC, MP3, AC3 (Dolby Digital), FLAC.

- Lossy compression loses some detail (MP4 with H.264); lossless keeps everything but makes huge files.

- Hardware acceleration: the GPU decodes and encodes video (NVENC, Quick Sync, AMD VCN), which saves CPU load and power.

- Optical media: DVD (480p), Blu-ray (1080p), Ultra HD Blu-ray (4K with HDR).

6. Software side

- Graphics drivers: needed for full performance and features. Download them from NVIDIA, AMD or Intel, or via Windows Update. Updating can fix crashes, glitches and game issues.

- Device Manager > Display adapters: shows the GPU and driver status. You can roll back a driver here.

- Task Manager > Performance > GPU: shows GPU usage, VRAM and temperature.

- dxdiag: shows DirectX version, GPU, VRAM and drivers, and tests.

- Display settings: resolution, refresh rate, HDR (Day 9).

- Linux: lspci | grep -i vga lists the GPU, nvidia-smi shows NVIDIA status, glxinfo shows OpenGL information.

- Free tools: GPU-Z (GPU specs), HWiNFO and MSI Afterburner (monitoring), 3DMark (benchmark), DDU (Display Driver Uninstaller) for a clean driver reinstall.

- BIOS/UEFI options: primary display (iGPU or PCIe), Above 4G Decoding and Resizable BAR for modern GPUs.

- Power and cooling: the GPU slows (throttles) at high temperatures. Keep fans and case airflow clean.

7. Installing a video card

- Shut down, unplug the PC, and use an ESD strap.

- Remove the slot covers and open the PCIe x16 slot clip.

- Press the card firmly into the slot until it clicks, then screw the bracket.

- Connect the PCIe power cables from the PSU (6, 8 or 12VHPWR pin as needed), using dedicated cables.

- Plug the monitor into the graphics card ports, not the motherboard ports.

- Install drivers from the manufacturer.

- Check Device Manager and Display settings.

Make sure the PSU wattage and connectors meet the card’s recommended requirements.

8. Common problems

| Problem | Likely cause | Fix |
| --- | --- | --- |
| No signal or black screen | Wrong input or port, loose cable, no GPU power cable, or card not seated | Check cable, port and power connectors; reseat the card |
| Artifacts (odd colours, lines, shapes) | Overheating, overclock, failing VRAM or GPU | Clean fans, reset clocks, update drivers; replace the card if it persists |
| Screen flickers or goes black during use | Bad driver, loose cable, power issue | Clean-install driver (DDU), check cable and PSU |
| Low resolution or no 4K option | Driver missing, old cable, port limits | Install drivers; use a cable and port that support the resolution |
| 4K only at 30 Hz | Old HDMI 1.4 port or cable | Use HDMI 2.0 or DisplayPort |
| No sound over HDMI or DisplayPort | Wrong playback device selected | Select the HDMI/DP audio device in Sound settings |
| Blank screen on streaming or Blu-ray content | HDCP failure | Use compliant cable, monitor or adapter; connect directly |
| Low game performance | Outdated drivers, thermal throttling, monitor connected to the motherboard, power-saving mode | Update drivers, check temperatures, plug into the GPU, set the correct GPU |
| Games use integrated GPU in a laptop | Wrong graphics preference | Settings > Display > Graphics: set the app to high performance |
| Driver crashes or “Display driver stopped responding” | Driver fault, overheating, overclock | Reinstall drivers, lower clocks, improve cooling |
| PC shuts down in games | PSU too weak or overheating | Check wattage and temperatures |
| Video lag or stutter | Weak CPU or GPU for the codec, no hardware acceleration | Enable hardware acceleration, update drivers, lower the resolution |
| Distorted picture on a VGA connection | Long or poor cable, wrong resolution | Use a shorter good cable, set native resolution, auto-adjust the monitor |

Exam points

- VGA introduced analog video at 640 x 480; the later SVGA, XGA and others follow its pattern.

- VGA is analog; DVI can be digital (D), analog (A) or both (I); HDMI and DisplayPort are digital and carry audio.

- HDMI 2.0 supports 4K at 60 Hz, HDMI 2.1 supports 4K at 120 Hz and 8K at 60 Hz.

- DisplayPort supports MST daisy-chaining and uses a locking connector.

- Full HD = 1920 x 1080, QHD = 2560 x 1440, 4K = 3840 x 2160, 8K = 7680 x 4320.

- Progressive (p) draws every line; interlaced (i) alternates odd and even lines.

- HDCP is copy protection for digital video.

- Integrated GPUs share system RAM; dedicated GPUs have their own VRAM.

- Codecs: H.264, H.265/HEVC, AV1. Containers: MP4, MKV, AVI.

- DirectX, OpenGL and Vulkan are graphics APIs.

- Plug the monitor into the graphics card, not the motherboard.

- Artifacts often mean an overheating or failing GPU.

- dxdiag shows DirectX and display information.

Day 11: Standard Thick Clients and Graphics/CAD/CAM Workstations (simple explanation)

Why this topic?

Not every computer is built for the same job. A custom PC configuration means choosing hardware to match what the user actually does. Today covers two types, and the next days cover the rest (audio/video, virtualization, gaming, home theatre, home server and thin client).

Analogy: buying a vehicle. A family car (thick client) suits daily use, while a heavy-duty truck (CAD workstation) is built for demanding jobs.

Part 1: Standard Thick Client

What is it?

A thick client (also called a fat client) is a normal, fully capable computer. It does its own processing and stores its own software and data locally, rather than depending on a server to do the work. Most office desktops and laptops are thick clients.

It is used for everyday tasks: documents, spreadsheets, email, web browsing, video calls, light photo editing and business applications.

Requirements

A standard thick client must:

- Meet the minimum or recommended requirements of the operating system (see below).

- Meet the requirements of the applications it will run.

- Be able to run on a network (wired or wireless) to reach servers, printers and the internet.

Windows 11 minimum requirements (good example)

| Component | Minimum |
| --- | --- |
| Processor | 1 GHz or faster, 2 or more cores, 64-bit compatible (supported CPU list) |
| RAM | 4 GB |
| Storage | 64 GB or more |
| Firmware | UEFI with Secure Boot capable |
| TPM | Version 2.0 |
| Graphics | DirectX 12 compatible with WDDM 2.0 driver |
| Display | HD (720p), larger than 9 inches diagonal |
| Other | Internet connection and a Microsoft account for Home edition setup |

Windows 10 needed only 1 GHz, 1 GB RAM (32-bit) or 2 GB (64-bit), and 16/20 GB storage. Minimum means it runs; recommended gives a smooth experience.

Typical thick client specification

| Part | Typical choice |
| --- | --- |
| CPU | Intel Core i3/i5 or AMD Ryzen 3/5, 4-6 cores |
| RAM | 8-16 GB |
| Storage | 256-512 GB SSD |
| Graphics | Integrated |
| Network | Gigabit Ethernet, Wi-Fi 6 |
| Display | Full HD monitor |
| OS | Windows 11 Pro, macOS, or Linux |

Pros and cons

| Pros | Cons |
| --- | --- |
| Works even if the server or network is down | Each PC needs its own updates, security and licences |
| Powerful for local applications | Data stored locally can be lost or stolen |
| Flexible for users | More expensive to buy and maintain than thin clients |
| Does not overload the server | Hardware becomes outdated |

Good practice: keep the OS and applications updated, use antivirus, back up important data, and use disk encryption on laptops.

Part 2: Graphics and CAD/CAM Design Workstation

What is it?

A workstation is a high-performance computer built for professional, demanding work. A graphics/CAD/CAM workstation is used by designers, engineers and architects.

Terms:

- CAD (Computer-Aided Design): creating 2D drawings and 3D models of products, buildings or parts. Examples: AutoCAD, SolidWorks, Revit, Fusion 360, CATIA.

- CAM (Computer-Aided Manufacturing): turning a design into machine instructions (such as G-code) that control CNC machines, laser cutters and 3D printers.

- Graphics design: image and illustration work with Photoshop, Illustrator, CorelDRAW and Blender.

- 3D rendering and modelling: Maya, 3ds Max, Blender and similar tools turn a model into a realistic image.

Hardware requirements and why

| Component | What to choose | Reason |
| --- | --- | --- |
| CPU | Multi-core, high clock speed (Intel Core i7/i9, Xeon, AMD Ryzen 9, Threadripper) | Modelling relies on fast single-core speed; rendering uses many cores |
| GPU (video card) | Professional/workstation card such as NVIDIA RTX A-series (formerly Quadro) or AMD Radeon Pro, with large VRAM | Smooth display of large 3D models, stable certified drivers, accurate rendering |
| RAM | 32-64 GB or more, ECC on serious workstations | Large models and scenes need lots of memory; ECC prevents silent errors |
| Storage | Fast NVMe SSD for OS and projects, large drive or RAID for archives, plus backups | Large files must load quickly; RAID or backups protect work |
| Monitor(s) | High-resolution (QHD or 4K), IPS, colour-accurate; often 2 or more screens | Detail, accurate colour and more workspace |
| PSU | High-wattage, high-efficiency unit | Powerful CPU and GPU draw a lot |
| Cooling | Strong cooling with good airflow | Heavy sustained loads create heat |
| Case/board | Roomy case, motherboard with many slots and RAM capacity | Room for large GPUs, extra drives and memory |
| UPS | Recommended | Protects work from power cuts |

Professional GPU vs gaming GPU

|  | Workstation GPU (RTX A-series, Radeon Pro) | Gaming GPU (GeForce, Radeon RX) |
| --- | --- | --- |
| Drivers | ISV-certified and tested with CAD/design software | Optimised for games |
| Reliability | Built for continuous use, ECC VRAM on many models | Fine for general use |
| VRAM | Often larger | Varies |
| Price | Much higher | Lower |
| Best for | Professional CAD, simulation, engineering | Games, and many creative uses |

ISV (Independent Software Vendor) certification means the software maker tested the card and driver for stability with their application.

Display and colour

- Colour accuracy: matters for print and product design. Check colour gamut (sRGB, Adobe RGB for print, DCI-P3 for video) and Delta E (lower is more accurate; below 2 is excellent).

- Calibration: use a colorimeter or spectrophotometer to create a colour profile (ICC profile) so screens show true colour.

- Multi-monitor: one screen for the model and another for toolbars or reference drawings.

- Large or high-resolution screens make fine detail easier to see.

Input and output devices

- Graphics tablet / pen display (Wacom): pressure-sensitive pen for drawing and retouching.

- 3D mouse (for example a 3Dconnexion SpaceMouse): controls 3D model movement smoothly.

- Precision mouse and a full keyboard with a numeric keypad.

- Output: large-format plotters for engineering drawings, 3D printers for prototypes, CNC machines and laser cutters (CAM), and colour-accurate photo printers.

- Scanners: 3D scanners and large-format scanners for digitising objects and plans.

Software side

- Operating system: usually Windows 11 Pro or Pro for Workstations (supports more CPUs, more RAM, ReFS and faster file sharing). Linux and macOS are also common in some fields.

- Certified drivers: install the professional driver (for example NVIDIA Enterprise/Studio drivers) instead of generic ones.

- Software requirements: check each application’s official hardware requirements. Many CAD programs rely on single-thread CPU speed for modelling, while rendering benefits from many cores or the GPU.

- File formats: DWG and DXF (AutoCAD), STEP and IGES (exchange between CAD programs), STL and OBJ (3D printing and models), G-code (CNC and 3D printer instructions).

- Version control and backups: large design files should be backed up regularly, ideally with versions and an off-site copy.

- Licensing: CAD software is expensive, and licences can be per user, network or subscription.

Backup and protection

Design work is valuable. Use:

- Regular backups (local and cloud).

- RAID 1/5/10 for the working drive (this is not a replacement for backup).

- A UPS to prevent corruption during a power loss.

- Antivirus and strong passwords.

Common problems

| Problem | Likely cause | Fix |
| --- | --- | --- |
| Software slow or lagging with large models | Not enough RAM, VRAM or weak GPU/CPU | Upgrade RAM or GPU, close other programs, use a certified card |
| Display glitches or crashes in CAD | Wrong or old graphics driver | Install the certified professional driver |
| Wrong colours on screen vs print | Uncalibrated monitor or wrong colour profile | Calibrate with a colorimeter, use the correct profile |
| Overheating and throttling during rendering | Poor cooling or dust | Clean the case, add fans, check thermal paste |
| Random restarts under load | Weak PSU | Use a high-quality unit with enough wattage |
| Large files open slowly | Slow or full drive | Use an NVMe SSD and keep space free |
| Software will not start or says unsupported | GPU or OS does not meet requirements | Check the vendor’s hardware compatibility list |
| Plotter or printer prints the wrong scale | Wrong page size or scaling setting | Match drawing scale and paper size in print settings |
| Lost work after a power cut | No UPS or autosave | Use a UPS and enable autosave |

Thick client vs CAD workstation

|  | Standard thick client | CAD/graphics workstation |
| --- | --- | --- |
| Purpose | General office tasks | Design, engineering, 3D work |
| CPU | Mid-range | High-end, many cores |
| RAM | 8-16 GB | 32-64 GB or more, often ECC |
| GPU | Integrated | Professional dedicated card |
| Storage | 256-512 GB SSD | Fast NVMe plus large storage, RAID/backup |
| Monitor | Standard | High-resolution, colour-accurate, often several |
| Cost | Low to moderate | High |

Exam points

- A thick client processes and stores data locally and meets the OS and application requirements.

- A thin client (Day 13) depends on a server.

- CAD creates designs; CAM produces machine instructions for manufacturing.

- CAD/graphics workstations need a powerful multi-core CPU, a high-end professional GPU with plenty of VRAM, maximum RAM, and fast storage.

- Workstation GPUs use ISV-certified drivers for stability.

- ECC RAM corrects memory errors.

- Multiple colour-accurate IPS monitors are common, calibrated with a colorimeter.

- Plotters, 3D printers and CNC machines are typical output devices.

- Windows 11 requires TPM 2.0, UEFI with Secure Boot, 4 GB RAM and 64 GB storage.

- G-code instructs CNC machines and 3D printers.

- A UPS protects against power loss.