System Settings

What are System Settings?

System settings are the options that configure how the computer hardware and operating system behave. They control things like display, sound, network, power, date and time, user accounts and startup. They exist at two levels:

- Hardware level: BIOS/UEFI (CMOS) setup

- Operating system level: Windows Settings and Control Panel

1. BIOS/UEFI Setup (Hardware-Level Settings)

What is BIOS?

BIOS (Basic Input Output System) is firmware stored on a chip on the motherboard. It runs first when you switch on the computer, tests the hardware (POST: Power On Self Test), and then loads the operating system. UEFI (Unified Extensible Firmware Interface) is the modern replacement, with a mouse-driven graphical interface, faster boot and support for drives larger than 2 TB.

Its settings are saved in CMOS memory, which is kept alive by the CMOS battery (CR2032).

How to Enter BIOS Setup

Press a key repeatedly right after switching on the computer:

| Brand | Common Key |
| --- | --- |
| Most desktops | Del or F2 |
| Dell | F2 or F12 |
| HP | F10 or Esc |
| Lenovo | F1 or F2 |
| Asus | Del or F2 |
| Acer | F2 or Del |
| Boot menu only | F12, F9 or Esc |

On Windows 10/11 you can also go to Settings → System → Recovery → Advanced startup → Restart now → Troubleshoot → UEFI Firmware Settings.

Main BIOS/UEFI Settings

| Section | Settings |
| --- | --- |
| Main / System Information | Date and time, BIOS version, CPU type, RAM size, serial number |
| Boot | Boot order (HDD, SSD, USB, DVD, network), fast boot, boot mode (UEFI or Legacy) |
| Advanced | CPU features (virtualization, hyper-threading), SATA mode (AHCI/RAID/IDE), USB and onboard device settings |
| Security | Supervisor/user password, Secure Boot, TPM |
| Power | Power-on after power failure, wake-on-LAN, ACPI sleep states |
| Overclocking / Performance | CPU multiplier, memory speed (XMP/EXPO profile), voltages, fan speed curves |
| Monitoring (H/W Monitor) | CPU temperature, fan speed, voltages |
| Save and Exit | Save changes, discard changes, Load Optimized (Default) Settings |

Important BIOS Settings Explained

- Boot order: decides which device the computer starts from. Change it to boot from a USB drive when installing an OS.

- SATA mode: AHCI is recommended for modern drives. Changing it after installing Windows can cause a boot failure.

- Secure Boot: allows only trusted, signed operating systems to start (required for Windows 11).

- TPM (Trusted Platform Module): hardware security feature used for encryption (BitLocker); required for Windows 11.

- Virtualization (Intel VT-x, AMD-V/SVM): needed to run virtual machines and Android emulators.

- XMP / EXPO: one-click profile that runs RAM at its rated speed instead of the default slower speed.

- BIOS password: prevents others from changing settings or booting the PC.

- Reset to defaults: fixes problems caused by wrong settings.

Clearing/Resetting CMOS

If the BIOS is locked or the computer will not start after a wrong setting:

- Switch off and unplug the computer.

- Remove the CMOS battery for a minute, then reinstall it, or use the CLR_CMOS jumper on the motherboard.

- Reconnect and start the computer. Settings return to defaults.

Updating BIOS (Flashing)

A BIOS update adds CPU support and fixes bugs. Download it from the motherboard manufacturer’s site and use the built-in flash tool or Windows utility. Never switch off the computer during an update, because it can make the motherboard unusable.

2. Operating System Settings (Windows)

Ways to Open System Settings

- Settings app: press Windows + I

- Control Panel: Start → type “Control Panel” (older, but still used)

- Run command: press Windows + R and type a command (see the table below)

- Right-click Start menu: quick links to Device Manager, Disk Management and more

Windows Settings App: Main Categories

| Category | What You Can Do |
| --- | --- |
| System | Display, sound, notifications, focus, power and battery, storage, multitasking, clipboard, remote desktop, About |
| Bluetooth and devices | Add Bluetooth devices, printers and scanners, mouse, touchpad, USB, AutoPlay |
| Network and internet | Wi-Fi, Ethernet, VPN, airplane mode, mobile hotspot, proxy, data usage |
| Personalization | Background, colors, themes, lock screen, taskbar, Start menu, fonts |
| Apps | Installed apps, default apps, startup apps |
| Accounts | Your info, sign-in options (PIN, password), family, email accounts |
| Time and language | Date and time, language, region, keyboard, typing |
| Gaming | Game mode, captures |
| Accessibility | Text size, magnifier, narrator, captions, high contrast |
| Privacy and security | Permissions (camera, microphone, location), Windows Security, device encryption |
| Windows Update | Check for and install updates, update history, pause updates |

Key System Settings in Detail

A. Display Settings

Resolution, scale, refresh rate, orientation, brightness, night light, HDR and multiple displays (covered in the previous topic).

B. Sound Settings

- Choose the output device (speakers, headphones, HDMI) and input device (microphone)

- Adjust master volume and per-app volume (Volume mixer)

- Enable or disable devices and run the sound troubleshooter

C. Power and Battery Settings

- Power modes: Best power efficiency, Balanced, Best performance

- Screen and sleep timeouts: when the display turns off and the PC sleeps

- Battery saver: limits background activity at low charge

- Lid and power button actions (Control Panel → Power Options): sleep, hibernate or shut down

| State | Meaning |
| --- | --- |
| Sleep | Low power; work stays in RAM; resumes quickly |
| Hibernate | Saves RAM contents to disk and powers off completely |
| Shut down | Closes everything and powers off |
| Restart | Closes everything and reboots |
| Fast startup | Hybrid of shutdown and hibernate for quicker boot |

D. Storage Settings

- View disk usage by category

- Storage Sense: automatically deletes temporary files

- Manage drives, create and format volumes (also via Disk Management)

- Change where new apps and files are saved

E. Date and Time Settings

- Set time zone (India uses IST, UTC+5:30), automatic time sync and date/time format

- Wrong time often means the CMOS battery is weak

F. Language and Region

- Add display languages and keyboard layouts

- Set regional formats (date, currency, number)

G. Network Settings

- Connect to Wi-Fi or Ethernet

- View IP address, DNS and MAC address (ipconfig /all in Command Prompt)

- Set a static IP or DHCP (automatic)

- Configure VPN, proxy, hotspot and firewall

H. Accounts and Security Settings

- Create administrator and standard user accounts

- Set a password, PIN, fingerprint or face sign-in

- User Account Control (UAC): asks permission before system changes

- Windows Security: antivirus, firewall, device security, BitLocker encryption

I. Apps and Startup

- Uninstall or repair programs (Settings → Apps, or Control Panel → Programs and Features)

- Disable unnecessary startup apps to speed up boot (Task Manager → Startup tab)

- Set default apps for browser, mail, media

J. Windows Update

- Install security and feature updates and drivers

- Set active hours and pause updates temporarily

- Roll back a problematic update if needed

K. Privacy Settings

Control which apps may use the camera, microphone, location and diagnostic data.

3. Control Panel and System Tools

Important Control Panel Items

| Item | Purpose |
| --- | --- |
| System | View processor, RAM, Windows edition, computer name |
| Device Manager | View and manage hardware and drivers |
| Programs and Features | Install, uninstall, repair software |
| Power Options | Create and edit power plans |
| Network and Sharing Center | Manage connections and adapters |
| User Accounts | Add or change users |
| Date and Time | Clock and time zone |
| Sound | Playback and recording devices |
| Mouse and Keyboard | Pointer speed, buttons, repeat rate |
| Printers / Devices and Printers | Add and manage printers |
| Windows Defender Firewall | Allow or block network traffic |
| Backup and Restore / File History | Backup files |
| Troubleshooting | Automatic fixes for common problems |

Device Manager

Shows all hardware installed. You can:

- Update driver, roll back driver, disable or uninstall a device

- See problems marked by a yellow triangle (driver problem) or a down-arrow (disabled)

Open it with Windows + X → Device Manager, or type devmgmt.msc in Run.

Useful Run Commands

| Command | Opens |
| --- | --- |
| control | Control Panel |
| msconfig | System Configuration (boot options, startup, services) |
| devmgmt.msc | Device Manager |
| diskmgmt.msc | Disk Management |
| services.msc | Services |
| taskmgr | Task Manager |
| regedit | Registry Editor (use with care) |
| ncpa.cpl | Network Connections |
| appwiz.cpl | Programs and Features |
| powercfg.cpl | Power Options |
| sysdm.cpl | System Properties |
| cmd | Command Prompt |
| msinfo32 | System Information |
| dxdiag | DirectX and hardware diagnostics |
| winver | Windows version |
| mstsc | Remote Desktop |

System Properties (sysdm.cpl)

Has tabs for:

- Computer Name: change the PC name and join a domain or workgroup

- Hardware: Device Manager

- Advanced: performance settings (visual effects, virtual memory/page file), user profiles, startup and recovery

- System Protection: System Restore points

- Remote: enable Remote Desktop

Task Manager (Ctrl + Shift + Esc)

- Processes: see CPU, memory, disk and network use by each app; end frozen apps

- Performance: live graphs for CPU, RAM, disk, Wi-Fi and GPU

- Startup apps: enable or disable programs that run at boot

- Users and Services: manage background tasks

System Configuration (msconfig)

Controls Safe Mode boot, boot options and which services start. Use it for troubleshooting.

4. Other Important System Configuration Topics

Virtual Memory (Page File)

Part of the disk that Windows uses as extra RAM when RAM is full. Adjust it at System Properties → Advanced → Performance → Settings → Advanced → Virtual memory. Letting Windows manage it automatically is usually best.

Drivers

Small programs that let the OS communicate with hardware. Keep graphics, chipset, network and audio drivers updated through Windows Update or the manufacturer’s website.

System Restore and Recovery

- Restore point: a snapshot of system files and settings you can return to if something breaks

- Reset this PC: reinstalls Windows (keep or remove files)

- Safe Mode: starts Windows with minimal drivers for troubleshooting (hold Shift while clicking Restart, then Troubleshoot → Startup Settings)

- Advanced startup / Windows Recovery Environment: repair tools, command prompt, startup repair

Environment Variables

Named values used by the OS and programs (such as PATH, TEMP). Found under System Properties → Advanced → Environment Variables.

Registry

A database of system and application settings (regedit). Changing it incorrectly can damage Windows, so back it up first.

Disk Management

Initialize, partition, format and assign drive letters. Use diskmgmt.msc.

Security Best Practices

- Keep Windows and antivirus updated

- Use strong passwords and enable a BIOS password if needed

- Use a standard account for daily work

- Turn on the firewall and BitLocker (if available)

- Back up important data regularly

5. System Settings in Other Operating Systems

| OS | Where Settings Are Found |
| --- | --- |
| Windows | Settings app (Win + I) and Control Panel |
| macOS | System Settings (Apple menu → System Settings) |
| Linux (Ubuntu) | Settings app; command-line tools such as nmcli, timedatectl, systemctl |
| Android | Settings app (gear icon) |
| iOS/iPadOS | Settings app |